%N tmcvt

%D Convert time-multiplexed polarization visibilities to Stokes.

%P rjs

%: uv analysis

%B

TMCVT converts time-multiplexed polarization observations to

Stokes parameters, applying any leakage correction in the process.

For an observation where polarizations are time multiplexed

(i.e. four polarization states are not simultaneously measured),

there is no simple way to convert between raw polarization

visibilities and Stokes parameters. This task operates by finding

the four polarizations measurements at a given instant which result in

the minimum time mismatch. It then uses this set in the conversion process.

%A vis

The names of the input uv data sets. Wildcards are supported.

No default.

%A line

Standard line parameter, with standard defaults. See the help on

"line" for more information.

%A select

Standard visibility data selection. See the help on "select" for

more information. The default is to select all data.

%A out

The name of the output uv data set. No default.

%A stokes

Several Stokes parameters of the output. Possible values are

i, q, u and v. There is no default - this parameter must be

specified.

%A interval

Time interval tolerance, in minutes. This sets the maximum mismatch

between polarization correlations that can be tolerated. The default

is 10 minutes,

%A options

Extra processing options. Several options can be given,

separated by commas. Minimum match is supported.

nopol     Do not apply polarization leakage correction.

nocal     Do not apply antenna gain correction.

nopass    Do not apply bandpass calibration correction.

pseudo    By default, tmcvt generates a visibility stream where

the error from time mismatches is minimized for each output

Stokes visibility. This usually results with different

Stokes visibilities for a given baseline having different

time tags. The "pseudo" option causes tmcvt to generate an output

dataset where all the Stokes visibilities for a given baseline

are pseudo-simultaneous (same time tag). This results in larger

errors, but may be more convenient if you must form an output

image where the beams are the same for the different Stokes

parameters, or where you are going to written out to a FITS

file.