%N smatbpass

%D Removes time-dependent bandpass ripples (antenna-based).

%P jhz

%: calibration

%B

SmaTbpass is a MIRIAD task which removes time-dependent bandpass

ripples. SmaTbpass reads multiple files (bpfile) containing

antenna-based bandpass solved using either smamfcal or mfcal

in different time intervals. The bandpasses are interpolated

or extrapolated to a time with either linear fit from

the two nearby data points or orthogonal polynomial

fit from all the data points in the input files.

The antenna-based bandpass in each of the multiple input

files must be IDENTICAL in spectral format, i.e. they must have

the same number of channele, the same number of spectral

chunks, and the same polarizations. The data used for solving

for bandpass in the multiple files must be observed in the same

track as the observations of the target sources in the vis files.

The output contains the data that have been applied with

the bandpass corrections.

%A bpfile

Root name of input files. Files must be named as

bpfile_i for the ith file, e.g. mybpass_1, mybpass_2 ... mybpass_n.

Each of the n bpfiles contains a bandpass solved from an independent

time interval using either smamfcal or mfcal.  No default.

NOTE: The calibration tables (bandpass, gains, freqs) must present

in the input datasets.

%A nfiles

The number of bandpass files (bpfile) to read.

%A vis

The name of the input uv data sets in which the visibility

will be applied for bandpass corrections. Several can be given (wild

cards are supported) but the spectral configuration(correlator setup)

is identical through out the files. No default.

%A bptime

The time interval, in minutes, to interpolate/extrapolate the

bandpass solutions. The default is 60 minutes.

%A options

This gives extra processing options. Several options can be given,

each separated by commas. They may be abbreivated to the minimum

needed to avoid ambiguity. Possible options are:

nocal       Do not apply the gains file. By default, UVAVER

applies the gains file in copying the data.

opolyfit    Do least-square fit to the time variations in

bandpasses given in all the bpfile_i files with an

orthogonal polynomial of degree n input from

Keyword: polyfit; n upto 10 is supported.

Default: linear fit from two nearby time points.

cross       Apply the bandpass to the cross-hand polarization

visibilities in addition to the parallel ones.

Default: to the parallel ones only.

%A polyfit

polyfit gives a degree of orthogonal polynomial in least-sqaure

fit to the time variaton of the bandpass. Default: 3 or cubic.

polyfit upto 10 is supported.

%A out

The name of the output uv data set. No default.