%N taco

%D Dynamic scheduling program.

%P mchw

%: misc

%B

TACO is a scheduling program to sort a prioritized list into

an observing schedule using an atmopheric model or

phase monitor data to estimate the atmospheric fluctuations.

%A in

Input prioritized list. No default.

The format is telescope dependent. For Hatcreek the format is:

project, ra[hhmm], dec[dddmm], atmax[millimeter], lstmax

Format: project*33,2i2,1x,i3,i2,3x,f4.2

column(64)=lstmax

Lines begining with `#' are not used.

'atmax' is the maximum atmospheric phase rms. Default=0.3 mm

(1/10 wave at 3mm wavelength) is a reasonable threshold

without Selfcal. Larger values can be used with Selfcal.

Smaller than 0.1 mm corresponds to very good seeing at Hatcreek.

For each project, 'lstmax' specifies the maximum number

of repeats for each hour of lst. The default is 1 repeat.

'lstmax' provides a quantized mechanism for specifying

the HA range wanted.

'F' denotes that uv-coverage is not important.

To sample the complete HA range, the integral of 'lstmax'

over the available HA range should equal the maxtime.

The UT range can also be specified explicitly.

Keywords specify some parameters which could be given

for each project, and vice-versa.

%A telescop

name of telescope. Default=HATCREEK.

Get telescope parameters, such as 'latitude'.

%A elevlim

Elevation limit. Default=10 degrees, zero wait time.

Time to wait for the source to rise can be changed by setting

elevlim below the actual elevation limit for the observation.

%A maxtime

maximum scheduled time for each project. Default = 8 hours.

%A mintime

Minimum interval for which a project can be scheduled.

(or 80% of time above elevlim, whichever is smaller)

Default = 1 hour.

%A interval

LST scheduling interval used to re-evaluate schedule.

Default = 2 hours.

%A wait

Interval to wait before re-evaluating schedule when

no project can be scheduled. Default=0.1 hours.

%A lst

Starting LST in hours, given as a real number. e.g. 12.3.

%A stop

LST stop time for schedule, given as a real number which

can be larger than 24.0 to represent more than one day.

Default = lst + interval, i.e. only one project will be scheduled.

If stop is greater than lst + interval

then TACO advances the LST and schedules the next project.

%A phfile

Filename of phase monitor data used to estimate seeing.

The atmospheric phase rms is measured in a 5 min average on a

100 m baseline.  The default file is '$MIRCAT/ovro_phase.1996'.

%A phday

Starting time in phfile - decimal days since 01JAN

Used to estimate seeing. Default uses atmos model instead.

The unix command phday=`date '+%j'`  sets the current day number.

%A phfac

Phase factor used to multiply value in phfile to get path rms in

millimeters.  e.g. 12 GHz phase monitor needs

phfac = 300/12/360 = 0.069     Default=1.0.

%A atmos

Three values giving the mean and rms atmospheric fluctuations

in millimeters, and the atmospheric time constant in hours.

Default=0,0,1 i.e. perfect weather. atmos=.3,.1,0.05 is reasonable.

Typical values range from .1,.1,4 in good weather to 2,2,0.01 in bad.

%A log

Output log file of the observing schedule. Default is the terminal.

%A statin

Input filename containing the status of the observations.

TACO keeps track of the integration time

and HA range aquired for each project. This status can

be read in as a starting point for observations,

and/or written out as a record of the current status.

Lines begining with `#' are not used.

Default: no files are read or written.

%A statout

Output filename containing the status of the observations.

This file is updated after each observation scheduled.

%A options

Extra processing options. Possible values are:

debug    Print more output to monitor scheduled observations.

histo    Print histogram for atmospheric phase,

and run time statistics.