%N atlod

%D Convert an RPFITS file into Miriad uv format.

%P rjs

%: data transfer

%B

ATLOD is a MIRIAD task, which converts a uv data-set from the

RPFITS format to Miriad format.

%A in

Name of the input RPFITS files.  Several names can be given --

wildcard expansion is supported.  If a single name is given it

can be a raw tape device name (e.g. /dev/nrst0 in UNIX)

containing several files.  In this case, see the NFILES keyword

below.  There is no default.

%A out

Name of the output Miriad uv data-set.  No default.

%A ifsel

IF number(s) to select.  Default is all IFs.  For example,

if you observed with 5 GHz (frequency 1) and 8 GHz (frequency 2)

simultaneously, IF 1 would be the 5 GHz data and IF 2 would

be the 8 GHz data.  This now also lets you select zoom bands

using values greater than 2.  The freq 1 zoom bands come before

the freq 2 ones.

%A restfreq

The rest frequency, in GHz, for line observations.  By default,

the value in the RPFITS file is used.  Giving a value for the

"restfreq" parameter overrides the RPFITS file value.  If you do

set this parameter, you MUST give the same number of values as

the number of IFs written out.  A value of 0 is used for a

continuum observation.  For example, if you have two IFs, the

first of which is HI, and the second is continuum, use

restfreq=1.420405752,0

%A options

This gives extra processing options.  Several can be given,

separated by comas.

'birdie'  For CABB data:

CABB generates self-interference in a number of

channels across the spectrum due to 640 MHz clock

harmonics.  These birdies are fixed in channel

number for each CABB configuration.  The birdie

option currently knows about the 2048x1MHz continuum

mode and flags the affected channels, 100 band edge

channels on each side, and, at 20 and 13cm, the

unusable parts of the spectrum.

For pre-CABB data:

ATCA self-interference can corrupt channels at

integral multiples of 128 MHz.  The birdie option

flags these channels.  Additionally, in continuum

(33 channels/128MHz) mode, the birdie option dicards

every second channel, plus some edge channels.  The

channels discarded are those most likely affected by

the self-interference.  Discarding these channels

does not have a sensitivity penalty, because the

effective channel bandwidth is twice the channel

separation.

'reweight' For pre-CABB data: re-weight the lag spectrum to

eliminate the "Gibbs" phenomena in continuum data

(33 ch/128 MHz); ignored for all other data.

'compress' Write output data in compressed format.

'noauto'  Discard autocorrelation data.  The default is to

copy the autocorrelation data.

'nocross' Discard cross-correlation data.  The default is to

copy the cross correlation data.

'relax'   Do not flag visibilities based on SYSCAL

information.  The default is to flag visibilities if

they have not been preceded by a valid SYSCAL

record, or if the the values in the SYSCAL record

look bad.  SYSCAL values are checked for sampler

statistics being within 3% of 17.3%, or 0.5% of

50.0%, that the XY phase is within 10 degrees of its

running median, and that the XY amplitudes are

within 1 Jy or 10% of its running median.  The tests

for xy phase and amplitude are skipped for 3mm data

(as there is no noise calibration signal).

'mmrelax' This option is ignored, it is only present for

historical reasons.

'unflag'  Save any data that is flagged and also keep it's

syscal info.  By default ATLOD discards most data

that is flagged.

'opcorr'  Correct for atmospheric opacity.  This option is

possible for data measured after October 2003.

Because of the way system temperature is measured at

3mm (an "above atmosphere" measurement), it is not

appropriate for 3mm data.  This option is silently

ignored for 3mm data.  Generally it is only relevant

for 7mm and 12mm observations.  It does no harm (and

negligible good) for longer wavelengths.

'samcorr' Correct the pre-Dec93 data for incorrect sampler

statistics.  Since December 1993, sampler

corrections are performed online.  This option is

silently ignored for data measured after December

1993.

'xycorr'  Apply the on-line measurements of the XY phase.

This option is silently ignored for 3mm data

measured before October 2007.

'hanning' Hanning smooth spectra and drop every other channel

This option is ignored for 128-MHz, 33-channel data.

'bary'    Use the barycentre as the velocity rest frame.  The

default is to use the LSR frame.

'noif'    Do not map the simultaneous IFs to the IF axis.

By default ATLOD attempts to map the simultaneous

frequencies to the IF axis.  This will not be

possible if there are a different number of

polarisations in the different IFs.

'nopflag' If at least one polarisation of a set of 2 or 4

polarimetric spectra are bad, ATLOD normally flags

all of the polarisations.  Option nopflag changes

this so that only the nominally bad spectrum is

flagged.

'hires'   Treat bin data as measurements in the high time

resolution mode.  The output dataset contains no

bins, but instead appears as data measured with

small cycle times.

'pmps'    Undo `poor man's phase switching'.  This is an

obscure option that you should not generally use.

'single'  Assume input is a single dish RPFITS file (from

Parkes or Mopra).  This is usually used together

with option 'relax'.

'caldata' Save visibilities associated with certain system

calibrations.  Currently this consists of reference

pointing calibration and "paddle" measurements.

'nocacal' Flag data that atlod suspects is taken during a

CACAL scan.  There is potential for error in atlod

determining which data are and are not part of a

cacal scan.  Use this with caution.

'nopol'   Discard data that is not "parallel hand" Stokes

type.

'rfiflag' Flag channels at frequencies that are known to be

bad.  This uses the file rfiflag.txt in the current

directory or the default version in MIRCAT.  The

file should contain 2 frequencies per line, the

lower and upper end of the rfi in MHz.  Precede

comments with a '#'.

'notsys' Undo the online Tsys correction. Useful if RFI in

the tvchannel range has caused the corrections

to be very noisy. The resulting data will be in raw

counts scaled down by 10^6.

'nopack' Don't pack the two CABB autocorrelation bins into

one bin. The autocorrelation bin 1 and 2 will

contain noise cal OFF and ON data.

The default is to repack the info into bin 1:

XX and YY will contain GTP and SDO data in the real

and imaginary part; XY will contain the OFF data and

YX will contain the ON-OFF data (for XY). This will

allow recalculation of the Tsys and xyphase later.

%A nfiles

This gives one or two numbers, being the number of files to

skip, followed by the number of files to process.  This is only

useful when the input is a tape device containing multiple

files.  The default is 0,1 (i.e. skip none, process 1 file).

NOTE: Using this feature to skip many files on a tape is VERY

inefficient.  It is far faster to skip using operating system

commands.  When doing this, however, you should be aware is that

every RPFITS files consists of 3 tape files.  Thus you will want

to skip three times as many tape files as RPFITS files.  For

example, in UNIX, to skip 10 RPFITS files, use

mt -f /dev/nrst0 fsf 30

%A nscans

This gives one or two numbers, being the number of scans to

skip, followed by the number of scans to process.  NOTE: This

applies to all files read.  The default is to skip none and

process all scans.

%A nopcorr

This gives the number of frequencies to use per IF for opacity

correction. The default (2) is to use linear interpolation

across the spectrum. Maximum value is 32.

%A edge

Specify the percentage of edge channels the birdie option will

flag out. The default is 9.8 which will flag about 100 channels

from the bottom and top of a 2049 channel spectrum. This

parameter is only used if the birdie option is specified.

For concatenated spectra the width of a single zoom is used.

Note that noise and artefacts go up quickly towards the band

edge so making this much smaller will not gain you much.