%N ax

%D Convert ascii correlator data to Miriad for CARMA.

%P mchw

%: uv analysis

%B

AX is a MIRIAD task to convert ascii correlator data to Miriad.

%A in

The name of a text file containing the ascii correlation data.

%A out

This gives the name of the output Miriad uvdata file.

%A baseline

antenna pair used for this correlation data.

The antenna order determines the phase.

The Miriad convention is ant1 < ant2

if ant1 > ant2 then conjugate the data.

Default baseline=8,9

%A nchan

Number of spectral channels in this correlation data file.

Default nchan=15

%A ant

The name of a text file containing the position of the antennas.

There is no default. Each line of the text file gives three values,

being the x, y and z location of an antenna.

The antenna positions can be given in either a right handed

equatorial system or as a local ground based coordinates measured to the

north, east and in elevation. See the "baseunit" parameter to

specify the coordinate system. Some standard antenna configurations

can be found in $MIRCAT/*.ant for ATCA, BIMA and VLA telescopes.

The BIMA and VLA antenna tables, use with baseunit=1, whereas for

the ATCA, use baseunit=-51.0204.

The text file is free-format, with commas or blanks used to separate

the values. Comments (starting with #) can be included in the file.

%A baseunit

This specifies the coordinate system used in the antenna file.

A positive value for "baseunit" indicates an equatorial system,

whereas a negative value indicates a local system. The magnitude of

"baseunit" gives the conversion factor between the baseline units

used in the antenna file, and nanoseconds. The default value is +1,

which means that the antenna file gives the antenna position in an

equatorial system measured in nanoseconds.

E.g.    baseunit=-1 for topocentric coordinates in nanosecs,

baseunit=3.33564 for geocentric coordinates in meters.

%A longlat

Longitude and Latitude of array phase center. hh:mm:ss,dd:mm:ss format,

or as decimal hours and degrees.  Default for CARMA station 32

-118:08:29.927,37:16:49.37285.  Coordinates for some other

telescopes can be obtained from Miriad telepar task.

%A radec

Source right ascension and declination. These can be given in

hh:mm:ss,dd:mm:ss format, or as decimal hours and decimal

degrees. The default is to set RA = 0. and DEC = latitude.

%A time

Start time of the observation.  This is in the form

yymmmdd.ddd

or

yymmmdd:hh:mm:ss.s

The default is 05OCT04

With the unix date command you can use

date +%y%b%d:%H:%M:%S | tr '[A-Z]' '[a-z]'

Default is to take date from the ascii correlation data file.

%A inttime

Integration time for each record. Default inttime=10 seconds.

%A freq

LO1 frequency, bandwidth (GHz), and frequency band. Default 100.0,0.5,1

The spectral channels are centered at frequencies:

Band 1   USB  = LO1 + 1.0 + [1:nchan] * bandwidth/(nchan+1) GHz.

Band 1   LSB  = LO1 - 1.0 - [1:nchan] * bandwidth/(nchan+1) GHz.

Band 2   USB  = LO1 + 1.5 + [1:nchan] * bandwidth/(nchan+1) GHz.

Band 2   LSB  = LO1 - 1.5 - [1:nchan] * bandwidth/(nchan+1) GHz.

Band 3   USB  = LO1 + 2.0 + [1:nchan] * bandwidth/(nchan+1) GHz.

Band 3   LSB  = LO1 - 2.0 - [1:nchan] * bandwidth/(nchan+1) GHz.

Set the bandwidth to a negative value for LSB spectrum.