%N tvcln

%D Clark/Steer CLEAN with display of intermediates

%P rjs

%: deconvolution

%B

TVCLN is a MIRIAD task, which performs a hybrid Clark/Steer Clean

algorithm, which takes a dirty map and beam, and produces an output

map which consists of the Clean components. This output can be

input to SELFCAL to self-calibrate visibilities, or input to RESTOR

to produce a "clean" image. Optionally CLEAN can take as one of

its inputs a model of the deconvolved image. This model could be

from a previous CLEAN run, or from any of the other deconvolution

tasks (e.g. MAXEN).

%A map

The input dirty map, which should have units of Jy/beam. No

default.

%A beam

The input dirty beam. No default

%A model

An initial model of the deconvolved image. This could be the

output from a previous run of CLEAN, or the output of any of the

deconvolution tasks (e.g. MAXEN). It must have flux units of

Jy/pixel. The default is no model (i.e. a zero map).

%A out

The name of the output map. The units of the output will be

Jy/pixel. It can be input to RESTOR, CLEAN (as a model, to do

more cleaning), or to SELFCAL (for self-calibrating visibility

data).

%A gain

The minor iteration loop gain. Default is 0.1.

%A cutoff

CLEAN finishes when the absolute maximum residual falls below

CUTOFF. Default is 0.

%A niters

The maximum number of minor iterations. Clean finishes when

abs(NITERS) minor iterations have been performed. Clean may finish

before this point, however, if NITERS is negative and the absolute

maximum residual becomes negative valued, or if the cutoff level

(as described above) is reached.

%A region

This specifies the region to be Cleaned. The default is the largest,

centered region that it is safe to deconvolve.

%A phat

Cornwells prussian hat parameter. When cleaning extended sources,

CLEAN may produce a badly corrugated image. This can be suppressed

to some extent by cleaning with a dirty beam which has had a spike

added at its center (i.e. a beam that looks like a prussian hat).

PHAT gives the value of this spike, with 0 to 0.5 being good

values. Default is 0 (but use a non-zero value for extended

sources).

%A minpatch

The minimum patch size when performing minor iterations. Default

is 51, but make this larger if you are having problems with

corrugations. You can make it smaller when cleaning images which

consist of a pretty good dirty beam.

%A speed

This is the same as the speed-up factor in the AIPS APCLN.

Negative values makes the rule used to end a major iteration more

conservative. This causes less components to be found during a

major iteration, and so should improve the quality of the Clean

algorithm Usually this will not be needed unless you are having

problems with corrugations. A positive value can be useful when

cleaning simple point-like sources. Default is 0.

%A mode

This can be either "hogbom", "clark", "steer" or "any", and

determines the Clean algorithm used. If the mode is "any", then

CLEAN determines which is the best algorithm to use. The default

is "any".

%A clip

This sets the relative clip level in a Steer clean, values

typically being 0.75 to 0.9. The default is image dependent.

%A server

If this is set to the name of a TV device, then at the end of

each major cycle, the restored image is displayed on the TV.